|  | Portfolio Manager - What data is required?. |
| --- | --- |

In order for Portfolio Manager to calculate metrics about your property, you must provide several key pieces of information about your property's operation, in addition to your energy, water or waste data. The information required varies by the type of property and whether or not your property is eligible for an ENERGY STAR Score.

| Data Required for All Properties |  |
| --- | --- |
| Property Name |  |
| Property Address |  |
| Total Gross Floor Area of Property | sq ft/sq M |
| Irrigated Area | sq ft/sq M/Acres |
| Year Built/Planned for Construction Completion |  |
| Occupancy | % |
| Number of Buildings |  |

Helpful Hints for All Properties

Definitions for Property Use Details are available in the Portfolio Manager Glossary (in the Help section, or https://portfoliomanager.energystar.gov/pm/glossary).

Some properties may contain multiple Property Uses within a single building (e.g. office, data center, and parking; OR K-12 School and Swimming Pool). In most cases, EPA recommends you enter as few Property Uses as possible. More information about when to enter a separate Property Use is in this FAQ.

For properties with multiple tenants within the same property use (e.g. Office), these tenants should be entered separately only when the number of Weekly Operating Hours differs by more than 10 hours. For example, say an Office Building has a Gross Floor Area of 100,000 square foot (SF) where 75,000 SF operates 60 hours a week and 25,000 SF operates 80 hours a week. Enter these as two separate Property Uses (one 75,000 SF property and one 25,000 SF property).

Financial Office Uses

| Data Collected for Financial Office Uses |  |
| --- | --- |
| The following information is required to get an ENERGY STAR Score (if eligible): |  |
| Gross Floor Area |  |
| Weekly Operating Hours |  |
| Number of Workers on Main Shift |  |
| Number of Computers |  |
| Percent That Can Be Cooled |  |
| The following information is optional and not used to calculate a score; it may inform future analysis and score revisions by EPA and/or may help you manage and compare your properties: |  |
| Percent That Can Be Heated |  |

Definition for Financial Office

Financial Office refers to buildings used for financial services such as bank headquarters and securities and brokerage firms.

Gross Floor Area should include all space within the building(s), including but not limited to offices, trading floors, conference rooms and auditoriums, vaults, restrooms, kitchens used by staff, lobbies, atriums, fitness areas for staff, storage areas, stairways, and elevator shafts.

Helpful Hints for Financial Office

If more than 10 percent of the office's gross floor area on average was vacant through the last 12 months, enter the vacant space as a separate Property Use with zero for Weekly Operating Hours, Number of Workers on Main Shift and Number of Computers.

The Weekly Operating Hours value is the number of hours per week that the office is occupied by the majority of its occupants. It should not include hours when the building is occupied solely by maintenance/security personnel or HVAC run times when the building is not occupied by the majority of occupants.

The Number of Workers on Main Shift should be entered as the number of workers present on a site at the same time, not the total number of workers added up across all shifts during a day.

When determining the Number of Computers, do not count extra monitors or tablets.  For example, a desktop computer with 3 monitors would count as 1.  Similarly, a laptop computer with an external monitor would count as 1.

Meter Information

What's required to see metrics:

12 consecutive, complete months of bills if your energy or water is metered continuously.

At least one delivery if your energy is delivered in bulk quantities (e.g. filling a propane tank.)

Please copy this sheet as needed to account for all meters at your property.

| Basic Meter Information |  |
| --- | --- |
| Meter Name or ID |  |
| Meter Type (e.g. Electricity) |  |
| Units (e.g. kWh) |  |
| Date Meter Became Active |  |
| Date Meter Became Inactive |  |

You can use the form below to get ready to enter your data so you can see metrics, however you can create your property and set up your meters without entering your meter data. You can add bills later.

| Meter Bills |  |  |  |
| --- | --- | --- | --- |
| Start Date/Delivery Date | End Date (leave blank for deliveries) | Usage/Quantity | Cost (optional) |