|  | Portfolio Manager - What data is required?. |
| --- | --- |

In order for Portfolio Manager to calculate metrics about your property, you must provide several key pieces of information about your property's operation, in addition to your energy, water or waste data. The information required varies by the type of property and whether or not your property is eligible for an ENERGY STAR Score.

| Data Required for All Properties |  |
| --- | --- |
| Property Name |  |
| Property Address |  |
| Total Gross Floor Area of Property | sq ft/sq M |
| Irrigated Area | sq ft/sq M/Acres |
| Year Built/Planned for Construction Completion |  |
| Occupancy | % |
| Number of Buildings |  |

Helpful Hints for All Properties

Definitions for Property Use Details are available in the Portfolio Manager Glossary (in the Help section, or https://portfoliomanager.energystar.gov/pm/glossary).

Some properties may contain multiple Property Uses within a single building (e.g. office, data center, and parking; OR K-12 School and Swimming Pool). In most cases, EPA recommends you enter as few Property Uses as possible. More information about when to enter a separate Property Use is in this FAQ.

For properties with multiple tenants within the same property use (e.g. Office), these tenants should be entered separately only when the number of Weekly Operating Hours differs by more than 10 hours. For example, say an Office Building has a Gross Floor Area of 100,000 square foot (SF) where 75,000 SF operates 60 hours a week and 25,000 SF operates 80 hours a week. Enter these as two separate Property Uses (one 75,000 SF property and one 25,000 SF property).

Bank Branch Uses

| Data Collected for Bank Branch Uses |  |
| --- | --- |
| The following information is required to get an ENERGY STAR Score (if eligible): |  |
| Gross Floor Area |  |
| Weekly Operating Hours |  |
| Number of Workers on Main Shift |  |
| Number of Computers |  |
| Percent That Can Be Cooled |  |
| The following information is optional and not used to calculate a score; it may inform future analysis and score revisions by EPA and/or may help you manage and compare your properties: |  |
| Percent That Can Be Heated |  |

Definition for Bank Branch

Bank Branch refers to a commercial banking outlet that offers banking services to walk-in customers.

Gross Floor Area should include all space within the building(s), including banking areas, vaults, lobbies, atriums, kitchens used by staff, restrooms, conference rooms, storage areas, stairways, and elevator shafts. Drive-thru windows should be included in the open parking lot, not partially enclosed parking.

Helpful Hints for Bank Branch

When calculating Weekly Operating Hours, include only the hours the branch is open to the public.

Meter Information

What's required to see metrics:

12 consecutive, complete months of bills if your energy or water is metered continuously.

At least one delivery if your energy is delivered in bulk quantities (e.g. filling a propane tank.)

Please copy this sheet as needed to account for all meters at your property.

| Basic Meter Information |  |
| --- | --- |
| Meter Name or ID |  |
| Meter Type (e.g. Electricity) |  |
| Units (e.g. kWh) |  |
| Date Meter Became Active |  |
| Date Meter Became Inactive |  |

You can use the form below to get ready to enter your data so you can see metrics, however you can create your property and set up your meters without entering your meter data. You can add bills later.

| Meter Bills |  |  |  |
| --- | --- | --- | --- |
| Start Date/Delivery Date | End Date (leave blank for deliveries) | Usage/Quantity | Cost (optional) |